Work in progress overview report for feedback.

(Packaging of script not completed yet)

Program steps:

1. Import image of standard, „good“ dial as a template for comparison.

Script crops out circular region.

2. User labels regions of interest (ROI boxes) on template

For each new type of watch dial, the user labels the following regions.

- General ROI (green; e.g. marks around the edge which may be missing in a faulty dial)

- Numbers (blue; in case wrong number gets stamped on dial)

- Background (red; this gives a reference color for background subtraction).

These coordinates are compared between the template and the factory photo of the dial.

3. Factory images for comparison are imported and aligned with the template

Currently, this is done within the jupyter/python notebook using SIFT feature matching. For a final version, this would presumably be done directly by a script on the factory computer connected to the dial camera.

4. Numbers inside the ROI boxes from step 2 are classified and errors are flagged

This is done using a pre-trained neural network (relatively fast & accurate); the alternative is pytesseract (commercial OCR software, unfortunately quite slow and inaccurate).

Accuracy seems to be around 90%.

5. Non-numeric marks inside the “general” ROI boxes from step 2 are compared and major deviations from the template are marked.

This is done using thresholding based on background ROI color; checking for differences in mean intensity; and then marking the image at the location of the deviation.